Good — switching gears properly. Let's slow all the way down and build understanding brick by brick, not just chase the bug. I'll explain what each thing is, why it exists, and what its output tells us, before we touch the next command.

### Lecture 1: What ibv_rc_pingpong Actually Does

RC = Reliable Connection — one of several RDMA transport types (others: UC = Unreliable Connection, UD = Unreliable Datagram). RC is the one that behaves most like TCP conceptually: connection-oriented, guarantees delivery and ordering, retransmits on loss. It's the transport type most RDMA applications (including real GPU-cluster training frameworks) actually use, which is why it's the right one to test here.

ibv_rc_pingpong is a verbs-level test utility shipped with rdma-core. It does four things, in strict sequence:

- Opens the device you name with -d (e.g., dc1srv1rxe-real) — this is a handle to the RDMA hardware/software device, analogous to opening a socket.

- Creates a Queue Pair (QP) — this is the fundamental RDMA object. Unlike a TCP socket (one bidirectional stream), a QP is literally a pair of queues: a Send Queue and a Receive Queue. Every RDMA operation is posted as a "work request" onto one of these queues.

- Exchanges connection info out-of-band — this is the part you saw as local address: ... QPN ... PSN ... GID. QPN (Queue Pair Number) and PSN (Packet Sequence Number, RDMA's equivalent of a TCP sequence number) must be manually exchanged before the RDMA connection itself can be brought up — ibv_rc_pingpong does this exchange over a plain TCP socket on port 18515 behind the scenes. This is not RDMA traffic yet — it's bootstrapping.

- Transitions the QP through states — INIT → RTR (Ready To Receive) → RTS (Ready To Send) — then actually posts real RDMA send/receive work requests and measures them.

What you achieved today, precisely: step 3 succeeded — the QPN/PSN/GID exchange completed on both sides, meaning basic IP reachability and the out-of-band handshake worked. Step 4 partially succeeded (QP reached a connected state) but then a work request — an actual data-transfer operation — failed.

### Lecture 2: What "Work Request Flushed Error" Actually Means

Every RDMA operation (a send, a receive, a read, a write) is submitted as a Work Request (WR), posted onto the Send or Receive Queue. When it completes — successfully or not — a Completion Queue Entry (CQE) is generated, which the application reads to find out what happened.

IBV_WC_WR_FLUSH_ERR (status code 5, what you saw) has a very specific meaning: this particular work request didn't fail on its own — it was flushed because something earlier in the queue failed first, and the RDMA stack, upon hitting that earlier failure, discards ("flushes") every work request queued behind it rather than trying to process a queue that's now in an inconsistent state.

This means: wr_id 2's error is a symptom, not the disease. The actual root-cause failure is wr_id 0 or 1 — something that failed silently before this, and ibv_rc_pingpong's minimal output didn't surface it clearly.

### What We Just Ruled Out, and Why It Mattered to Check

MTU was a legitimate hypothesis — a mismatch between negotiated RDMA MTU and actual network path MTU is a real, common cause of this exact symptom. You just confirmed both devices report active_mtu: 4096, identical on both sides. This rules MTU out cleanly — not "probably fine," genuinely eliminated as a variable, because both sides agree.

### Next Diagnostic — One Command, With Full Explanation Before You Run It

ibv_rc_pingpong's default output is too sparse to see the real first failure. It has a verbose flag that shows every individual work completion, not just the final flush:

sh

ibv_rc_pingpong -d dc1srv1rxe-real -g 1 -n 1

What -n 1 does: by default, ibv_rc_pingpong runs 1000 iterations of send/receive. Reducing to -n 1 means only one single exchange is attempted — this makes the failure point unambiguous (it's the first one, not buried in a stream), and it's genuinely faster to interpret with a fresh eye.

Run this on dc1-srv1 (server) first, then dc2-srv1 (client) immediately after with the matching -n 1:

sh

ibv_rc_pingpong -d dc2srv1rxe-real -g 1 -n 1 10.10.10.10

Before you run either — question for you, to check understanding, not just to proceed: given what I just explained about WR Flush errors cascading from an earlier failure, why might reducing to a single iteration (-n 1) actually help us see the true first error more clearly, rather than just making the test "smaller"?

answer! this makes the failure point unambiguous (it's the first one, not buried in a stream), and it's genuinely faster to interpret with a fresh eye.

also what is RDMA and what is RXE then i run the above command dc1-server1:/ # dc1-server1:/ # ibv_rc_pingpong -d dc2srv1rxe-real -g 1 -n 1 10.10.10.10 local address: LID 0x0000, QPN 0x000012, PSN 0x27fd9a, GID ::ffff:10.10.10.11 Couldn't connect to 10.10.10.10:18515 dc1-server1:/ # dc2-server1:/ # dc2-server1:/ # dc2-server1:/ # ibv_rc_pingpong -d dc2srv1rxe-real -g 1 -n 1 10.10.10.10 local address: LID 0x0000, QPN 0x000013, PSN 0xa7a16e, GID ::ffff:10.10.10.11 Couldn't connect to 10.10.10.10:18515 dc2-server1:/ #

Thought for 5s

Good — that's the right idea, but let me sharpen it into a full answer since "faster to interpret" was doing a lot of hand-waving: with -n 1, there's only one WR posted, so if it fails, it's unambiguously that WR's own error — not a flush cascading from something three or four operations earlier that scrolled past unnoticed. Precision, not speed, is the real reason.

### Lecture 3: What RDMA Actually Is

RDMA = Remote Direct Memory Access. The core idea: let one computer write directly into (or read directly from) another computer's memory, without involving that remote computer's CPU or operating system kernel in the transfer.

Contrast with normal networking: when server A sends data to server B over TCP, server B's CPU has to receive the packet, copy it through the kernel's network stack, copy it again into an application buffer — multiple copies, multiple CPU interrupts, real CPU cycles spent just moving bytes around. With RDMA, server A's NIC writes the data directly into a pre-registered region of server B's memory — server B's CPU does nothing during the transfer. This is why RDMA matters enormously for AI training: thousands of GPUs need to exchange gradients constantly, and if every exchange burned CPU cycles on data copying, you'd be wasting the exact compute you paid for on GPUs.

This is also why the "GID" and pre-registration steps you've been doing matter — RDMA needs to know exactly which memory address on the remote side it's allowed to touch, negotiated in advance, unlike a normal socket where the OS mediates every read/write.

### Lecture 4: What RXE Is

RXE (also called Soft-RoCE) is a software emulation of an RDMA-capable NIC, implemented as a Linux kernel module (rdma_rxe, what you loaded on the host days ago). Real RDMA hardware (Mellanox/Nvidia ConnectX cards) does this memory-access magic in dedicated NIC silicon — no CPU involvement, genuinely offloaded. RXE fakes the same verbs API (the programming interface — ibv_devinfo, ibv_rc_pingpong, all the tools you've been using) on top of an ordinary Ethernet NIC, using the CPU to emulate what the hardware would normally do.

Practical consequence, worth being precise about: RXE lets you write and test real RDMA application code and verify the protocol (RoCEv2 — RDMA over Converged Ethernet, version 2) actually works end-to-end over your fabric. It does not give you the actual performance benefit (CPU offload) real hardware provides — the whole point of today's exercise is protocol/connectivity verification, not performance testing. This lines up exactly with what Workbook AI's Module A4 already flagged: soft-RoCE proves the wire protocol works, real DCQCN/hardware behavior stays out of reach regardless.

### Lecture 5: Where to Look When a Work Request Fails Silently

ibv_rc_pingpong deliberately gives minimal application-level error detail — that's by design, it's a lightweight test tool, not a debugger. When something fails below the application layer (memory registration, transport-level packet issues, checksum failures), the kernel usually logs something more specific, because rdma_rxe is a kernel module — its own internal errors get written to the kernel ring buffer.

Command to check, on the host (PA-ALT-N11), not inside a container — since containers share the host kernel, kernel-level RXE errors get logged at the host level, not inside the container's isolated view:

bash

dmesg | grep -i rxe | tail -30

What dmesg is: the kernel's own diagnostic message ring buffer — every kernel module, including rdma_rxe, writes error/warning messages here when something goes wrong internally, often with far more specific detail than what bubbles up to a userspace tool like ibv_rc_pingpong.

Why this matters conceptually: you've been debugging entirely from the application layer (verbs API, ibv_devinfo, ibv_rc_pingpong) — this is the first time we go one layer deeper, into the kernel module itself, to see what it actually logged about the failure.

Also check — is dc1-srv1's server pane still alive, or did it exit silently?

sh

# in the dc1-srv1 pane, just press Enter or check if you're back at a prompt

Run the dmesg check on the host and tell me the current state of dc1-srv1's pane — paste both.

### Lecture 6: What a RoCEv2 Packet Actually Looks Like

This matters for the next diagnostic step. RoCEv2 isn't its own separate wire protocol from scratch — it's InfiniBand transport headers, encapsulated inside a UDP packet, which itself rides inside a normal IP packet. Specifically: Ethernet → IP → UDP (destination port 4791) → InfiniBand Base Transport Header → payload. This is why RoCEv2 (unlike RoCEv1, which used raw Ethernet) can cross routed IP networks — including your VXLAN fabric — because it's just a UDP packet as far as your switches are concerned.

This means we can literally watch it with tcpdump, the same way you inspected ECN marks in Workbook AI's Module A3.

### Next Diagnostic — Capture the Actual Packets

On dc1-srv1 (the server side, which will be listening):

sh

tcpdump -i eth1 -n udp port 4791 &

The trailing & runs it in the background so you can still run the pingpong server in the same pane afterward.

Understanding check before you run it: given RoCEv2 rides on UDP port 4791, and we already know your VXLAN fabric successfully carries other UDP-encapsulated traffic (VXLAN itself is UDP-encapsulated too, port 4789) — what would it tell us if tcpdump shows zero packets captured during a test attempt, versus if it shows packets arriving but the test still fails? What's the diagnostic difference between those two outcomes?

Answer that, then we'll run the capture alongside a fresh pingpong attempt.

Redeploy:

bash

cd ~/network-labs/v01-dc-arista-gm-clab/scripts

./v11-dc-arista-deploy.sh

Once it completes, recreate both RXE devices — same commands as before, now second-nature:

dc1-srv1 pane:

sh

hostname

rdma link add dc1srv1rxe-real type rxe netdev eth1

dc2-srv1 pane:

sh

hostname

rdma link add dc2srv1rxe-real type rxe netdev eth1

Then, on dc1-srv1, confirm tcpdump is now present before doing anything else:

sh

which tcpdump

### Lecture 7: What strace Is, and Why It's the Right Next Tool

strace traces every system call a process makes — every request it sends to the kernel (open a file, send data on a socket, allocate memory, etc.) and what the kernel returned. This is the layer between the application (ibv_rc_pingpong) and the kernel module (rdma_rxe) — if the packet is being silently dropped somewhere in that handoff, strace should show us the exact syscall and its return code/error.

Soft-RoCE (RXE) actually sends its RDMA traffic by opening a real UDP socket internally and using it to transmit — this is worth knowing conceptually: even though you're using the RDMA verbs API at the application layer, underneath, RXE is still just a Linux program using sendto()/sendmsg() on a UDP socket. If that syscall itself fails or never gets called, that's your answer.

Run this on dc2-srv1 — trace the pingpong client process, filtering to network-related syscalls only (there'd be hundreds of irrelevant ones otherwise):

sh

apk add strace

strace -f -e trace=network -o /tmp/strace_output.txt ibv_rc_pingpong -d dc2srv1rxe-real -g 1 -n 1 10.10.10.10

What each flag does: -f follows child processes/threads (RXE work often happens in kernel worker threads spawned from the process). -e trace=network filters to only socket/network-related calls (socket, sendto, bind, connect, etc.) — without this filter, you'd get thousands of irrelevant lines. -o writes to a file instead of flooding your terminal, since you'll want dc1-srv1's server running normally in parallel without interleaved output confusing things.

Remember — dc1-srv1's server needs to be freshly started and waiting first, same coordination discipline as before.

Run apk add strace, confirm it installs, then set up the coordinated test with strace wrapping the client. Paste the install confirmation first.

### Lecture 8: Why the Actual RDMA Send Never Appears Here

You filtered with -e trace=network — deliberately, on my instruction, to reduce noise. But this was actually the wrong filter for what we needed, and it's worth understanding precisely why.

RDMA "sends" don't happen via normal socket syscalls like sendto(). When an application posts an RDMA work request (ibv_post_send), it doesn't call a network syscall directly — it writes a command to the RDMA verbs kernel interface via ioctl() (or a memory-mapped doorbell mechanism) on the /dev/infiniband/uverbsX device file. The actual packet construction and transmission happens inside the kernel, triggered by that ioctl(), not by userspace calling sendto() the way a normal socket program would.

My filter (trace=network) explicitly excludes ioctl() calls — so the exact syscall that would show us the RDMA work request being submitted was filtered out by the very command I gave you. That's a mistake on my part worth naming plainly, not glossing over — I picked the wrong filter for this specific question.

### Corrected Command — Trace Everything This Time

We need to see what happens between the successful connect() and the exit — that's where the real work request submission and failure live.

Restart dc1-srv1's server (the previous one already completed a connection and likely exited):

sh

# dc1-srv1 pane

nohup ibv_rc_pingpong -d dc1srv1rxe-real -g 1 -n 100 > /tmp/pingpong_server.log 2>&1 &

jobs

cat /tmp/pingpong_server.log

Confirm it shows only local address:, waiting.

Then, dc2-srv1 — full trace, no filter this time, so we catch the ioctl():

sh

strace -f -o /tmp/strace_full.txt ibv_rc_pingpong -d dc2srv1rxe-real -g 1 -n 1 10.10.10.10

After it finishes, don't dump the whole file — it'll be huge. Search specifically for the interesting parts:

sh

grep -E "ioctl|EINVAL|EPERM|ENOBUFS|ENOMEM|write\(|exited" /tmp/strace_full.txt | tail -40

This filters to ioctl calls (the real RDMA work submission), common error codes, and the exit line — the section of the trace most likely to contain the actual answer.

Run the server restart first, confirm it's waiting, then the full trace on dc2-srv1 — paste the grep output.

Found it. This is the actual, concrete answer — real evidence, not inference.

ioctl(3, RDMA_VERBS_IOCTL, 0x7ffed7ef93a0) = -1 ENOSPC (No space left on device)

ENOSPC on the very first RDMA_VERBS_IOCTL call. Every subsequent ioctl call returns = 0 (success) — meaning this one specific failure happened early, likely during a resource-allocation step (most plausibly memory registration — telling the kernel "here's a buffer, make it accessible for RDMA" — that's typically one of the first ioctl calls in the sequence, before send/receive work requests can even be posted).

### Lecture 9: What ENOSPC Actually Means Here — And Why It's Misleading

ENOSPC normally means "disk full" — but in the RDMA verbs subsystem, this error code gets reused to mean "a kernel resource pool is exhausted." Specifically, this is almost always caused by hitting the RLIMIT_MEMLOCK limit — the maximum amount of memory a process is allowed to "pin" (lock into physical RAM, preventing it from being swapped out).

Why pinning matters for RDMA: remember Lecture 3 — RDMA lets a remote NIC write directly into your memory without CPU involvement. For that to be safe, the memory region has to be locked so the kernel can't move or swap it out mid-transfer — if it did, the remote side would be writing into stale or wrong physical memory. Every RDMA operation requires registering a locked memory region first. If the process's memlock limit is too low (often defaulting to a small value like 64KB on standard Linux/Docker configs), even a modest RDMA buffer registration fails immediately with exactly this ENOSPC.

This connects directly back to something flagged in the Workbook AI appendix, days ago — IPC_LOCK capability. Recall the containerlab YAML edit added NET_ADMIN, SYS_ADMIN, and IPC_LOCK. IPC_LOCK is precisely the capability that allows a process to lock more memory than the default limit — but having the capability doesn't automatically raise the limit itself. Docker containers, even with IPC_LOCK granted, often still enforce a low ulimit -l (max locked memory) unless it's explicitly raised.

### Verify This Diagnosis Before Fixing Anything

sh

# on dc2-srv1

ulimit -l

Understanding check: given the default Docker/Linux memlock limit is commonly 64 (in KB), and RDMA memory registration for even a small test buffer needs to lock actual memory pages, what result do you expect this to show — and if it does confirm a low limit, why would adding IPC_LOCK as a capability in the YAML not have been sufficient on its own to fix this?

Run it, paste the result — if it confirms a low/default limit, the fix is a ulimits addition in the containerlab topology YAML (a memlock: -1 or a high explicit value, same "isolated single-variable edit" discipline as every fix today), not another capability.