InfiniBand Simulator Lab

Startup Runbook — ibsim + OpenSM

Host: PA-ALT-N11  ·  Working directory: ~/ibsim-lab

This runbook starts a real InfiniBand Subnet Manager (OpenSM) against a software-simulated fabric (ibsim). No real InfiniBand hardware is required or used. Follow the steps in order — the sequence matters.

# Before You Start

| Check | Command |
| --- | --- |
| Packages installed | which opensm ibsim |
| Working directory exists | ls ~/ibsim-lab/mytopo.net |

If either check fails, see “First-Time Setup” at the end of this document before continuing.

# Startup Procedure

## Step 1 — Open two terminals

You need two terminal windows/tabs open on PA-ALT-N11 for the whole time the lab is in use. Label them Terminal 1 and Terminal 2 to avoid confusion.

## Step 2 — Terminal 1: Start the simulated fabric

cd ~/ibsim-lab ibsim -r -l 12345 -s ./mytopo.net

Wait for this exact output before moving on:

Network simulator ready. sim>

⚠ DO NOT CLOSE THIS TERMINAL Leave Terminal 1 open and untouched for as long as the lab is in use. Closing it or pressing Ctrl+C ends the simulated fabric.

## Step 3 — Terminal 2: Start OpenSM

cd ~/ibsim-lab export SIM_HOST="Hca1" export IBSIM_SERVER_PORT=12345 export IBSIM_SERVER_NAME=localhost LD_PRELOAD=/usr/lib/x86_64-linux-gnu/umad2sim/libumad2sim.so opensm -g 0 -f ./osm.log

Confirm success — you must see all three of these lines:

Using default GUID 0x100001 Entering DISCOVERING state Entering MASTER state

If you see “Error from osm_opensm_init” instead, stop and go to Troubleshooting below — do not proceed to Step 4.

## Step 4 — Optional: run OpenSM in the background instead

If you don't want Terminal 2 tied up, add -B to the same command and it will run as a background daemon:

LD_PRELOAD=/usr/lib/x86_64-linux-gnu/umad2sim/libumad2sim.so opensm -g 0 -f ./osm.log -B

Verify it's genuinely running before trusting it:

ps aux ¦ grep opensm tail -5 ./osm.log

## Step 5 — Verify the fabric (any terminal)

sminfo ibnetdiscover

“sminfo” should show a MASTER state SM. “ibnetdiscover” should show Switch1, Hca1, and Hca2, matching the topology file.

# You're Ready

The simulated InfiniBand fabric is now live and under real Subnet Manager control. Use standard InfiniBand diagnostic tools (ibstat, ibnetdiscover, sminfo, and others from the infiniband-diags/ibutils packages) exactly as you would against real hardware.

# Shutting Down

- Terminal 2: Ctrl+C (or "pkill opensm" if running with -B)

- Terminal 1: Ctrl+C to stop ibsim

Nothing here persists — both processes must be started fresh (from Step 2) next time, including after any host reboot.

# Troubleshooting

| Symptom | Cause | Fix |
| --- | --- | --- |
| "Error from osm_opensm_init: IB_INSUFFICIENT_RESOURCES" | Missing/unwritable /var/cache/opensm directory | sudo mkdir -p /var/cache/opensm && sudo chown $USER:$USER /var/cache/opensm — then retry Step 3 |
| OpenSM exits immediately, no error shown | One of the three export commands wasn't run in the same terminal as the opensm command | Re-run all three export lines and the opensm command together, in Terminal 2 |
| "Failed to verify bind path" or similar on a fresh checkout | Packages not installed | See First-Time Setup below |
| Unsure if something is actually running | — | cat ~/ibsim-lab/osm.log — check the last few lines for the real error, don't guess |

💡 IF SOMETHING ELSE GOES WRONG Read ~/ibsim-lab/osm.log first — it usually contains the real error even when the terminal only shows a generic message. This single step resolved the hardest issue found while building this lab.

# First-Time Setup (New User or New Host Only)

Skip this section if packages are already installed and ~/ibsim-lab already exists.

sudo apt update sudo apt install -y opensm ibsim-utils infiniband-diags ibutils libibmad5 sudo mkdir -p /var/cache/opensm sudo chown $USER:$USER /var/cache/opensm mkdir -p ~/ibsim-lab cd ~/ibsim-lab cp /usr/share/doc/ibsim-utils/net-examples/net.1 ./mytopo.net

Then follow the Startup Procedure above from Step 1.